电子科技大学二零一零年至二零一一学年第二学期

“数字逻辑设计及应用”课程考试题（半期）（120分钟）考试日期 2013年5月11日
	一
	二
	三
	四
	五
	六
	七
	八
	九
	十
	总分
	评卷教师

	
	
	
	
	
	
	
	
	
	
	
	


I.  To fill the answers in the  “(      )”   ( 2’ X 15 = 30 )

1. [109.75] 10 = (                        )2 = (         ) 16 。

2. (2713)10 = (                        ) 2421 BCD.

3. ([-17] 10)’s signed-magnitude representation is (            ), and its 8-bit two’s complement representation is (            ).

4. If X’s signed-magnitude representation XSM is 111010, then (2X)’s 8-bit signed-magnitude representation is (         ), and (X/2)’s two’s complement representation is (            ). 

5. If the function
[image: image18.png]74x139

—D°—< 16 1Y0
1Y1

1A 1Y2

1B 1Y3

2G  2Y0

2v1

24 2v2

2B 2v3




, then the complement expression of the function F is 
[image: image2.wmf](,,)(        )

=

Õ

M

FABC

,（       ） and its’ dual expression is 
[image: image3.wmf](,,)(        )

=

Õ

d

M

FABC

（            ）.

6．CMOS three-state buffer has three-state outputs, HIGH, LOW and (          ).

7．If A two’s-complement =01011010, B two’s-complement =10001011, then whether occurs overflow of -A+B? (Yes or No) [          ].

8．If the Gray code is （1110101）gray，then its corresponding binary number code is (        ).

9．There is a static hazard for the logic circuit realized according to the logic expression
[image: image4.wmf]BC

AD

C

A

F

+

+

=

'

'

, then the corresponding logic expression with the same function but without the static hazard is (                                            ).
10．If VOLmax = 0.5 V, VOHmin = 2.8 V,  VILmax = 0.8 V,  VIHmin = 2.1 V for 74LS series , then DC noise margin is VNL = (        )V,  VNH = (         )V.

II.  There is only one correct answer in the following questions.( 3’ X 10=30 )

1. Which is the correct one in the following statements for a logic function? (      ).
A. The minimal expression may be the sum of products or product of the sums 

B. The minimal expression is the minimal form with product of sums 

C. The minimal expression is the minimal form with sum of products 

D. The minimal expression with sum of products is the same simply with the minimal expression with product of sums

2. Which of the following statements is the wrong one when perform arithmetic operations with two binary numbers? (      ). 

A. There is an overflow when the most significant bit (MSB) generates a carry out for the addition of two unsigned numbers. 

B. There is not an overflow for the addition of two complements with different MSB. 

C. There is an overflow when the MSB generates a carry out for the addition of two two’s complements.

D. The subtraction of two two’s complements can be realized with adder.

3. The static hazard of the combination logic circuit is caused by the reason that (      ).

A. the circuit is not the most simple one   
B. there is many outputs for the circuit  

C. different gate circuits are applied to the circuit     

D. there is propagation delays at different signal paths in the circuit

4. There is only one output is effective at the same time in which of the following devices (      ).

A. 74X138 Decoder  B. Seven-Segment Decoder    C. Multiplexer   D. 74X148 Encoder 

5. The truth table of a circuit is shown in the following table, then the logic expression of this circuit is (     ).
A.  F=C           B.  F=ABC       C.  F=AB+A’C       D.  F=BC+AC’ 
	A
	B
	C
	F
	
	A
	B
	C
	F

	0
	0
	0
	0
	
	1
	0
	0
	0

	0
	0
	1
	1
	
	1
	0
	1
	0

	0
	1
	0
	0
	
	1
	1
	0
	1

	0
	1
	1
	1
	
	1
	1
	1
	1


6. The relationship between the logic function F1 = ∑ABCD(2,4,5,7,9,14) and F2 = ∏ABCD(1,6,8,10,11,13) is (       ).
A. Duality       B. Equivalent       C. Shannon’s expansion      D. Complement
7. A priority encoder 74LS148’s input is: I0-L, I1-L, I2-L, I3-L, I4-L, I5-L, I6-L, I7-L，output is Y2-L,Y1-L,Y0-L.The inputs and output are all active-low. When active-low enable input S_L=0 ,and  I1-L=I5-L=I6-L =0, then Y2-L,Y1-L,Y0-L is (       ).
   A) 110      
B) 010     

C) 001    

D) 101 
8. In the following methods to describe a logic function, only (      ) has uniqueness.
A. the logic expression                                 B. the logic circuit diagram           
C. the truth table                                      D. the waveform diagram

9. The NAND gates in the case of the positive logic level is equivalent to the (     ) in the case of the negative logic level. 
A. the NAND gate                              B. the AND-OR gate            
C. the AND gate                                D. the NOR gate

10. Which of the following statements is the wrong one for combination circuits? (      ).
    A. Outputs depend only on its current inputs         

B. They do not generates dynamic hazards

   C. They can be formed by using integrated binary decoders and NAND gates

D. They contains no feedback loops

III.  Combinational Circuit Analysis And Design: [40’]

1.Given F(A,B,C,D)= ∑m (3,5,6,11,12,15)+∑d(1,7,13). Simplify the logic function F(A,B,C,D) into the minimal product expression using Karnaugh map, and write out NOR-NOR logic expression. (10’)

2. A combinational circuit is shown as below, assume propagation delay for each gate is equal to tpd. (10’)
(1) When A=B=D=0, C change from 0 to 1 or from 1 to 0, draw the timing diagram for Y1、Y2 and  Y.

(2) Analysis conditions that the static hazard may exit,  and write out the product-of-sum expression for the hazard-free.

[image: image1.wmf](,,)(1,2,4,7)

M

FABC

=

Õ

[image: image9.png]



3.A combinational circuit is shown as below, which contains a 74x151 multiplexer and a inverter with 4 inputs, 1-bit output. (10’)

(1) Write out the logic expression of the output F(S1,S0,A,B).

(2)When S1S0=01 and S1S0=10, describe the logic function of the circuit.

[image: image10.png]/)




[image: image11.png]74x151
R
—d EN
A A
S0 B
S1 C
* D0 Y

1.




4.Design a converter circuit with signed magnitude (x2x1x0) to 2’s complement (z2z1z0) using a 74x139 dual 2-to-4 decoder and needful gates, and draw the logic diagram. (10’)

[image: image12.png]74x139

— 1A
— 1B

—q 2G

1Y0
1Y1
1Y2
1Y3

2Y0
2Y1
2Y2
2Y3

7777 97797





参考解答
电子科技大学二零一零年至二零一一学年第二学期

“数字逻辑设计及应用”课程考试题（半期）（120分钟）考试日期 2013年5月11日
	一
	二
	三
	四
	五
	六
	七
	八
	九
	十
	总分
	评卷教师

	
	
	
	
	
	
	
	
	
	
	
	


I.  To fill the answers in the  “(      )”   ( 2’ X 15 = 30 )

1. [109.75] 10 = (    1101101.11     )2 = (  6D.C   ) 16 。

2. (2713)10 = (  0010110100010011  ) 2421 BCD.

3. ([-17] 10)’s signed-magnitude representation is (  10010001   ), and its 8-bit two’s complement representation is (   11101111    ).

4. If X’s signed-magnitude representation XSM is 111010, then (2X)’s 8-bit signed-magnitude representation is (  10110100  ), and (X/2)’s two’s complement representation is (  11110011    ). 

5. If the function
[image: image5.wmf](,,)(1,2,4,7)

M

FABC

=

Õ

, then the complement expression of the function F is 
[image: image6.wmf](,,)(        )

=

Õ

M

FABC

,（0，3，5，6） and its’ dual expression is 
[image: image7.wmf](,,)(        )

=

Õ

d

M

FABC

（1,2,4,7）.

6．CMOS three-state buffer has three-state outputs, HIGH, LOW and (   Hi-Z        ).

7．If A two’s-complement =01011010, B two’s-complement =10001011, then whether occurs overflow of -A+B? (Yes or No) [   Yes    ].

8．If the Gray code is （1110101）gray，then its corresponding binary number code is (   1011001     ).

9．There is a static hazard for the logic circuit realized according to the logic expression
[image: image8.wmf]BC

AD

C

A

F

+

+

=

'

'

, then the corresponding logic expression with the same function but without the static hazard is (  F=A’C’+AD+BC+C’D+A’B或F=A’C’+AD+BC+ BD   ).
10．If VOLmax = 0.5 V, VOHmin = 2.8 V,  VILmax = 0.8 V,  VIHmin = 2.1 V for 74LS series , then DC noise margin is VNL = (   0.3     )V,  VNH = (  0.7  )V.

II.  There is only one correct answer in the following questions.( 3’ X 10=30 )

1. Which is the correct one in the following statements for a logic function? (  A  )

A. The minimal expression may be the sum of products or product of the sums 

B. The minimal expression is the minimal form with product of sums 

C. The minimal expression is the minimal form with sum of products 

D. The minimal expression with sum of products is the same simply with the minimal expression with product of sums

2. Which of the following statements is the wrong one when perform arithmetic operations with two binary numbers? (  C  ) 

A. There is an overflow when the most significant bit (MSB) generates a carry out for the addition of two unsigned numbers. 

B. There is not an overflow for the addition of two complements with different MSB. 

C. There is an overflow when the MSB generates a carry out for the addition of two two’s complements.

D. The subtraction of two two’s complements can be realized with adder.

3. The static hazard of the combination logic circuit is caused by the reason that (  D   ).

A. the circuit is not the most simple one   
B. there is many outputs for the circuit  

C. different gate circuits are applied to the circuit     

D. there is propagation delays at different signal paths in the circuit

4. There is only one output is effective at the same time in which of the following devices (   A   ).

A. 74X138 Decoder  B. Seven-Segment Decoder    C. Multiplexer   D. 74X148 Encoder 

5. The truth table of a circuit is shown in the following table, then the logic expression of this circuit is (  C   ).
A.  F=C           B.  F=ABC       C.  F=AB+A’C       D.  F=BC+AC’ 
	A
	B
	C
	F
	
	A
	B
	C
	F

	0
	0
	0
	0
	
	1
	0
	0
	0

	0
	0
	1
	1
	
	1
	0
	1
	0

	0
	1
	0
	0
	
	1
	1
	0
	1

	0
	1
	1
	1
	
	1
	1
	1
	1


6. The relationship between the logic function F1 = ∑ABCD(2,4,5,7,9,14) and F2 = ∏ABCD(1,6,8,10,11,13) is (  A  ).
A. Duality       B. Equivalent       C. Shannon’s expansion      D. Complement
7. A priority encoder 74LS148’s input is: I0-L, I1-L, I2-L, I3-L, I4-L, I5-L, I6-L, I7-L，output is Y2-L,Y1-L,Y0-L.The inputs and output are all active-low. When active-low enable input S_L=0 ,and  I1-L=I5-L=I6-L =0, then Y2-L,Y1-L,Y0-L is (      C      ).
   A) 110      
B) 010     

C) 001    

D) 101 
8. In the following methods to describe a logic function, only (   C   ) has uniqueness.
A. the logic expression                                 B. the logic circuit diagram           
C. the truth table                                      D. the waveform diagram

9. The NAND gates in the case of the positive logic level is equivalent to the (  D   ) in the case of the negative logic level. 
A. the NAND gate                              B. the AND-OR gate            
C. the AND gate                                D. the NOR gate

10. Which of the following statements is the wrong one for combination circuits? (  B  )

    A. Outputs depend only on its current inputs         

B. They do not generates dynamic hazards

   C. They can be formed by using integrated binary decoders and NAND gates

D. They contains no feedback loops

III.  Combinational Circuit Analysis And Design: [40’]

1.Given F(A,B,C,D)= ∑m (3,5,6,11,12,15)+∑d(1,7,13). Simplify the logic function F(A,B,C,D) into the minimal product expression using Karnaugh map, and write out NOR-NOR logic expression. (10’)

[image: image13.png]1/2 74x139 truth table

GLB A | Y3LY2L YlLLY0L
1 XX 11 1 1
0 00 1 1 1 0
0 01 1 1 0 1
010 1 0 1 1
011 o 1 1 1





最简表达式
Fminimal-product (A,B,C,D)=(B+C)(B+D)(A+C+D)(A/+C/+D)

或Fminimal-product (A,B,C,D)=(B+C)(B+D)(A+C+D)(A/+B/+C/)

“或非-或非“表达式

FNOR-NOR (A,B,C,D)=[(B+C)/+(B+D)/+(A+C+D)/+(A/+C/+D)/]/
或FNOR-NOR (A,B,C,D)=[(B+C)/+(B+D)/+(A+C+D)/+(A/+B/+C/)/]/
2. A combinational circuit is shown as below, assume propagation delay for each gate is equal to tpd. (10’)
(1) When A=B=D=0, C change from 0 to 1 or from 1 to 0, draw the timing diagram for Y1、Y2 and  Y.

(2) Analysis conditions that the static hazard may exit,  and write out the product-of-sum expression for the hazard-free.

[image: image14.png]CD

ADN00 01 11 10
00/(0} d)| 1 o
01 1(d]|1
114 'E_
-
w00 0| 1 |\





[image: image15.png]



[image: image16.png]CD
AB

00 01 11 10

00: g

01

L

11

10




1.定时图
2.  Y(A,B,C,D)=(A+B+C/)(C+D)

    静态冒险存在的条件：A=B=D=0，C变化。
   Y hazard-free (A,B,C,D)=(A+B+C/)(C+D)(A+B+D)

[image: image17.png]Truth table

X; X X

e — R — R — R}
Ll — T — I — )
—_ e D e D = D

N
5

N
—-

N
<

e — R — R — R}
)
R — I — N )




3.A combinational circuit is shown as below, which contains a 74x151 multiplexer and a inverter with 4 inputs, 1-bit output. (10’)

(1) Write out the logic expression of the output F(S1,S0,A,B).

(2)When S1S0=01 and S1S0=10, describe the logic function of the circuit.


1. F的表达式
F(S1,S0,A,B)= A/S1/S0/+ABS0+AS1S0/+BS1 或

F(S1,S0,A,B)=A/(S1/S0/)+(AB)(S1/S0)+(A+B)(S1S0/)+(A(B)(S1S0)
2.电路功能描述


S1S0=01，F(A,B)=AB

S1S0=10，F(A,B)=A+B

4.Design a converter circuit with signed magnitude (x2x1x0) to 2’s complement (z2z1z0) using a 74x139 dual 2-to-4 decoder and needful gates, and draw the logic diagram. (10’)


答案：
z2(x2,x1,x0)= ∑m(4,5,6,7)

z1(x2,x1,x0)=∑m(2,3,5,6)

z0(x2,x1,x0)=∑m(1,3,5,7)


_1234567893.unknown

_1234567895.unknown

_1234567896.unknown

_1234567897.unknown

_1234567894.unknown

_1234567891.unknown

_1234567892.unknown

_1234567890.unknown

