电子科技大学2011 -2012学年第 二  学期期 末 考试 A  卷

课程名称：_数字逻辑设计及应用__ 考试形式： 闭卷   考试日期： 20 12 年 07 月  02 日

考试时长：_120___分钟

课程成绩构成：平时  30   %， 期中  30   %， 实验    0     %， 期末  40   %

本试卷试题由___七__部分构成，共__6___页。

| 题号 | 一 | 二 | 三 | 四 | 五 | 六 | 七 | 八 | 九 | 十 | 合计 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 得分 |  |  |  |  |  |  |  |  |  |  |  |

得 分

I.  Fill out your answers in the blanks  (3’ X 10=30’)

1. The inputs of a full-adder are A =1, B=1, Cin=1. Then the output Cout is (        	).

2. If an 74x148 priority encoder has its 0, 2, 4, and 6 inputs at the active level, the active LOW binary output is (          ).

3. If an J-K flip-flop with J=1 and K=1 has a 10kHz clock input, the Q output is a (        ) Hz square wave.

4. A sequential circuit whose output depends on the state and inputs is called a (             ) state machine.

5. If we observed the waveforms of output Q1、Q2、Q3 of three flip-fl0ps of a counter as the Figure 1, the modulo of the counter should be (         ).

CP

Q1

Q2

Q3

Figure 1

6. State/output table for a sequential circuit is shown as table 1. X is input and Zn is output. Assume that the initial state is S0, if the input sequence is X=01011101, the output sequence should be (                   ).

| Table 2 Transition/output table |  |  |
| --- | --- | --- |
| X Q2Q1 | 0 | 1 |
| 00 01 10 11 | 10/1 00/0 01/0 10/0 | 01/0 10/0 00/1 00/0 |
| Q2*Q1*/Y |  |  |

| Table 1 State/output table |  |  |
| --- | --- | --- |
| X Sn | 0 | 1 |
| S0 S1 S2 S3 | S0/1 S1/0 S2/0 S2/0 | S1/1 S2/0 S0/0 S0/0 |
| Sn+1/Zn |  |  |

7.Transition/output table for a sequential circuit is shown in Table 2, X is input and Y is output, the sequential circuit is a modulus (          ) up/down counter.

8. A 4-bit linear feedback shift-register (LFSR) counter with no self-correction can have (      ) normal states.

9. A serial sequence generator by feedback shift registers 74x194 is shown in Figure 2, assume the initial state is Q2Q1Q0=110, the feedback function LIN = Q2/Q1/ + Q2Q0/, the output sequence in Q2 is (                 ).

Figure 2

10. When the input is 01000000 of an 8 bit DAC, the corresponding output voltage is 2V.  The output voltage range for the DAC is 0 ~  (               )V.

得 分

II. Please select the only one correct answer in the following questions.(2’ X 5=10’)

1. If	 a 74x85 magnitude comparator has ALTBOUT=AGTBOUT=0, AEQBOUT=1 on its outputs, the cascading inputs are (	           ).

A) ALTBIN=0, AEQBIN=0, AGTBIN=0            B) ALTBIN=0, AEQBIN=1, AGTBIN=0

C) ALTBIN=1, AEQBIN=0, AGTBIN=1         	 D) ALTBIN=1, AEQBIN=1, AGTBIN=1

2. For an edge-triggered D flip-flop, (	         	) is correct.

A) a change in the state of the flip-flop can only at a clock pulse edge

B) the state that the flip-flop goes to depends on the D input

C) the output follows the input at each clock pulse

D) all of these answers

3. An asynchronous counter differs from a synchronous	 in (             ).

A) the number of states in its sequence          B) the method of clocking

C) the type of flip-flop used                  D) the value of the modulus

4. A modulus-10 Johnson counter requires (	             	).

A) ten flip-flops		B) four flip-flops		C) five flip-flops		D)  twelve flip-flops

5. The capacity of a memory that has 10 bits address bus and can store 8 bits at each address is (            ).

A) 1024               B) 8192              C) 80       	 D) 256

得 分

III.  A D Latch and a D Flip-flop are shown in Figure 3. The waveforms of a clock CP and an input D are shown in Figure 4. Assume the initial state of Q is 1. Try to draw the waveforms of Q of the D Latch and the D Flip-flop. Ignore the delay of the circuit. (15’ )

Figure 3

Q1=X(Q1+Q0)Q0=X(Q1+Q0)Y=XQ1Q0

Figure 4

得 分

IV.  Analyze the clocked synchronous state machine shown in Figure 5. (15’)

- Write out the excitation equations, the state transition equations and the output equations of the circuit. (5’)

- Fill out the transition/output table, and draw the state transition/output diagram, determine whether the circuit can start itself automatically. (8’)

- Describe the logic function of the FSM.(2’)

Figure 5

The transition /output table:

| Q3Q2Q1 | Q3*Q2*Q1* | Y |
| --- | --- | --- |
| 000 |  |  |
| 001 |  |  |
| 010 |  |  |
| 011 |  |  |
| 100 |  |  |
| 101 |  |  |
| 110 |  |  |
| 111 |  |  |

得 分

V.  Design a variable modulus counter only with 74x163, which will be a module-6 counter when the input control signal M = 1 or a module-8 counter when M = 0. The states of module-6 counter should be（0,11,12,13,14,15）.The states of module-8 counter should be (0,9,10,11,12,13,14,15）. (10’)

- Please write out the input equations of LD_L, A, B, C, D. ( 5’)

- Draw the circuit linked line of 74x163. (5’)

74X163的功能表

| 输入 |  |  |  | 当前状态 | 下一状态 | 输出 |
| --- | --- | --- | --- | --- | --- | --- |
| CLR_L | LD_L | ENT | ENP | QD QC QB QA | QD* QC* QB* QA* | RCO |
| 0 | X | X | X | X X X X | 0 0 0 0 | 0 |
| 1 | 0 | X | X | X X X X | D C B A | 0 |
| 1 | 1 | 0 | X | X X X X | QD QC QB QA | 0 |
| 1 | 1 | X | 0 | X X X X | QD QC QB QA | 0 |
| 1 | 1 | 1 | 1 | 0 0 0 0 | 0 0 0 1 | 0 |
| 1 | 1 | 1 | 1 | 0 0 0 1 | 0 0 1 0 | 0 |
| 1 | 1 | 1 | 1 | 0 0 1 0 | 0 0 1 1 | 0 |
| 1 | 1 | 1 | 1 | 0 0 1 1 | 0 1 0 0 | 0 |
| 1 | 1 | 1 | 1 | …………. | ………….. | 0 |
| 1 | 1 | 1 | 1 | 1 1 1 1 | 0 0 0 0 | 1 |

得 分

VI.  Fill out the state/output table for a Mealy machine that can detect the pattern 01010 or 01110 . The output Z=1 when the pattern is detected. Your model should also be able to detect overlapping sequences. (10’)

State/output table

| State meaning | S | X |  |
| --- | --- | --- | --- |
|  |  | 0 | 1 |
| Initial state | A |  |  |
|  | B |  |  |
|  | C |  |  |
|  | D |  |  |
|  | E |  |  |
|  | F |  |  |
|  | G |  |  |
|  |  | S*, Z |  |

得 分

VII.  The transition equations and output equitation of a clocked synchronous state machine is shown as bellows. Please fill out the transition/output table and the state/output table. The states Q1 Q0=00~11, use the state name A~D. [10']

Q1=X(Q1+Q0)Q0=X(Q1+Q0)Y=XQ1Q0

Transition/output table

| Q1Q0 | X |  |
| --- | --- | --- |
|  | 0 | 1 |
| 00 |  |  |
| 01 |  |  |
| 10 |  |  |
| 11 |  |  |
|  | Q1*Q0*,Y |  |

State/output table

| S | X |  |
| --- | --- | --- |
|  | 0 | 1 |
| A |  |  |
| B |  |  |
| C |  |  |
| D |  |  |
|  | S*,Y |  |

参考解答

电子科技大学2011 -2012学年第 二  学期期 末 考试 A  卷

课程名称：_数字逻辑设计及应用__ 考试形式： 闭卷   考试日期： 20 12 年 07 月  02 日

考试时长：_120___分钟

课程成绩构成：平时  30   %， 期中  30   %， 实验    0     %， 期末  40   %

本试卷试题由___六__部分构成，共__6___页。

| 题号 | 一 | 二 | 三 | 四 | 五 | 六 | 七 | 八 | 九 | 十 | 合计 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 得分 |  |  |  |  |  |  |  |  |  |  |  |

得 分

I.  Fill out your answers in the blanks  (3’ X 10=30’)

1. The inputs of a full-adder are A =1, B=1, Cin=1. Then the output Cout is (     1   	).

2. If an 74x148 priority encoder has its 0, 2, 4, and 6 inputs at the active level, the active LOW binary output is (    001     ).

3. If an J-K flip-flop with J=1 and K=1 has a 10kHz clock input, the Q output is a (   5k    ) Hz square wave.

4. A sequential circuit whose output depends on the state and inputs is called a (  Mealy   ) state machine.

5. If we observed the waveforms of output Q1、Q2、Q3 of three flip-fl0ps of a counter as the Figure 1, the modulo of the counter should be (   6      ).

CP

Q1

Q2

Q3

Figure 1

6. State/output table for a sequential circuit is shown as table 1. X is input and Zn is output. Assume that the initial state is S0, if the input sequence is X=01011101, the output sequence should be (   11000100    ).

| Table 2 Transition/output table |  |  |
| --- | --- | --- |
| X Q2Q1 | 0 | 1 |
| 00 01 10 11 | 10/1 00/0 01/0 10/0 | 01/0 10/0 00/1 00/0 |
| Q2*Q1*/Y |  |  |

| Table 1 State/output table |  |  |
| --- | --- | --- |
| X Sn | 0 | 1 |
| S0 S1 S2 S3 | S0/1 S1/0 S2/0 S2/0 | S1/1 S2/0 S0/0 S0/0 |
| Sn+1/Zn |  |  |

7.Transition/output table for a sequential circuit is shown in Table 2, X is input and Y is output, the sequential circuit is a modulus (    3     ) up/down counter.

8. A 4-bit linear feedback shift-register (LFSR) counter with no self-correction can have (  15  ) normal states.

9. A serial sequence generator by feedback shift registers 74x194 is shown in Figure 2, assume the initial state is Q2Q1Q0=110, the feedback function LIN = Q2/Q1/ + Q2Q0/, the output sequence in Q2 is (   110100   ).

Figure 2

10. When the input is 01000000 of an 8 bit DAC, the corresponding output voltage is 2V.  The output voltage range for the DAC is (  0 ~ 7.97   )V.

得 分

II. Please select the only one correct answer in the following questions.(2’ X 5=10’)

1. If	 a 74x85 magnitude comparator has ALTBOUT=AGTBOUT=0, AEQBOUT=1 on its outputs, the cascading inputs are (	B  ).

A) ALTBIN=0, AEQBIN=0, AGTBIN=0            B) ALTBIN=0, AEQBIN=1, AGTBIN=0

C) ALTBIN=1, AEQBIN=0, AGTBIN=1         	 D) ALTBIN=1, AEQBIN=1, AGTBIN=1

2. For an edge-triggered D flip-flop, (	D	) is correct.

A) a change in the state of the flip-flop can only at a clock pulse edge

B) the state that the flip-flop goes to depends on the D input

C) the output follows the input at each clock pulse

D) all of these answers

3. An asynchronous counter differs from a synchronous	 in (      B       ).

A) the number of states in its sequence          B) the method of clocking

C) the type of flip-flop used                  D) the value of the modulus

4. A modulus-10 Johnson counter requires (	C	).

A) ten flip-flop		B) four flip-flop		C) five flip-flop		D)  twelve flip-flop

5. The capacity of a memory that has 10 bits address bus and can store 8 bits at each address is ( B  ).

A) 1024               B) 8192              C) 80       	 D) 256

得 分

III.  A D Latch and a D Flip-flop are shown in Figure 3. The waveforms of a clock CP and an input D are shown in Figure 4. Assume the initial state of Q is 1. Try to draw the waveforms of Q of the D Latch and the D Flip-flop. Ignore the delay of the circuit. (15’ )

Figure 3

Figure 4

Answer:

In the waveform of the output, the delay of the gate circuit has been taken into consideration.

得 分

IV.  Analyze the clocked synchronous state machine shown in Figure 5. (15’)

- Write out the excitation equations, the state transition equations and the output equations of the circuit. (5’)

- Fill out the transition/output table, and draw the state transition/output diagram, determine whether the circuit can start itself automatically. (8’)

- Describe the logic function of the FSM.(2’)

Figure 5

Solution：

（1）The excitation  equation:

D1=Q3’,  D2=Q1,  D3=Q1Q2

The state equation and the output equation:

Q1*=D1=Q3’,  Q2*=D2=Q1,  Q3*=D3=Q1Q2,  Y=(Q3Q1’)’=Q3’+Q1

（2）The transition /output table:

| Q3Q2Q1 | Q3*Q2*Q1* | Y |
| --- | --- | --- |
| 000 | 001 | 1 |
| 001 | 011 | 1 |
| 010 | 001 | 1 |
| 011 | 111 | 1 |
| 100 | 000 | 0 |
| 101 | 010 | 1 |
| 110 | 000 | 0 |
| 111 | 110 | 1 |

The sate transition /output diagram:

Since all the unused states can enter the asserted state after several states transition, it can start itself automatically.

(3) This circuit is a mould-5 counter.

得 分

V.  Design a variable modulus counter only with 74x163, which will be a module-6 counter when the input control signal M = 1 or a module-8 counter when M = 0. The states of module-6 counter should be（11,12,13,14,15,0）.The states of module-8 counter should be (9,10,11,12,13,14,15,0）. (10’)

- Please write out the input equations of LD_L, A, B, C, D. ( 5’)

- Draw the circuit line linked of 74x163.(5’)

74X163的功能表

| 输入 |  |  |  | 当前状态 | 下一状态 | 输出 |
| --- | --- | --- | --- | --- | --- | --- |
| CLR_L | LD_L | ENT | ENP | QD QC QB QA | QD* QC* QB* QA* | RCO |
| 0 | X | X | X | X X X X | 0 0 0 0 | 0 |
| 1 | 0 | X | X | X X X X | D C B A | 0 |
| 1 | 1 | 0 | X | X X X X | QD QC QB QA | 0 |
| 1 | 1 | X | 0 | X X X X | QD QC QB QA | 0 |
| 1 | 1 | 1 | 1 | 0 0 0 0 | 0 0 0 1 | 0 |
| 1 | 1 | 1 | 1 | 0 0 0 1 | 0 0 1 0 | 0 |
| 1 | 1 | 1 | 1 | 0 0 1 0 | 0 0 1 1 | 0 |
| 1 | 1 | 1 | 1 | 0 0 1 1 | 0 1 0 0 | 0 |
| 1 | 1 | 1 | 1 | …………. | ………….. | 0 |
| 1 | 1 | 1 | 1 | 1 1 1 1 | 0 0 0 0 | 1 |

【Solution】：the input equations:

LD_L=Q3

D=A=1

C=0

B=M

得 分

VI.  Write out the state/output table for a Mealy machine that can detect the pattern 01010 or 01110 . The output Z=1 when the pattern is detected. Your model should also be able to detect overlapping sequences. (10’)

| State meaning | S | X |  |
| --- | --- | --- | --- |
|  |  | 0 | 1 |
| Initial state | A | B, 0 | A, 0 |
| Got 0 | B | B, 0 | C, 0 |
| Got 01 | C | D, 0 | E, 0 |
| Got 010 | D | B, 0 | F, 0 |
| Got 011 | E | B, 0 | G, 0 |
| Got 0101 | F | D, 1 | E, 0 |
| Got 0111 | G | B, 1 | A, 0 |
|  |  | S*, Z |  |

得 分

VII.  The transition equations and output equitation of a clocked synchronous state machine is shown as bellows. (10’) Please fill out the transition/output table and the state/output table. [10']

Q1=X(Q1+Q0)Q0=X(Q1+Q0)Y=XQ1Q0

Transition/output table

| Q1Q0 | X |  |
| --- | --- | --- |
|  | 0 | 1 |
| 00 | 00,0 | 01,0 |
| 01 | 00,0 | 10,0 |
| 10 | 00,0 | 11,0 |
| 11 | 00,0 | 11,1 |
|  | Q1*Q0*,Y |  |

State/output table

| S | X |  |
| --- | --- | --- |
|  | 0 | 1 |
| A | A,0 | B,0 |
| B | A,0 | C,0 |
| C | A,0 | D,0 |
| D | A,0 | D,1 |
|  | S*,Y |  |